Automatic Hand Sanitizer Dispenser

Description:

The Automatic Hand Sanitizer Dispenser is a contactless hygiene system designed to dispense sanitizer automatically when a hand is detected.
This project helps reduce physical contact and prevents the spread of germs.
It uses an ultrasonic sensor to detect the presence of a hand and a servo motor to press or release the sanitizer container mechanism.

Block Diagram:

Pinout / Circuit Connection:

Ultrasonic Sensor TRIG → Arduino Pin 9
Ultrasonic Sensor ECHO → Arduino Pin 10
Ultrasonic Sensor VCC → 5V
Ultrasonic Sensor GND → GND

Servo Motor Signal → Arduino Pin 6
Servo Motor VCC → 5V
Servo Motor GND → GND

Power Supply: 9V Battery (connected to Arduino VIN)

Components Required:

Arduino Uno / Nano
Ultrasonic Sensor (HC-SR04)
Servo Motor (SG90 or equivalent)
9V Battery
Hand Sanitizer Container
Jumper wires
Breadboard

Working Principle:

The project works on the principle of ultrasonic distance measurement.
The ultrasonic sensor emits high-frequency sound waves and measures the time taken for the echo to return.
When a hand is placed within a predefined distance, the Arduino detects it.
The Arduino then rotates the servo motor, which presses the sanitizer container to dispense sanitizer.
After a short delay, the servo returns to its original position.

Operation (Step-by-Step):

- Mount the ultrasonic sensor near the sanitizer outlet.

- Connect all components as per the circuit diagram.

- Upload the Arduino code to the board.

- Power ON the system using a 9V battery.

- When a hand comes close to the sensor, distance reduces.

- Arduino detects the hand and activates the servo motor.

- Servo motor presses the sanitizer container.

- Sanitizer is dispensed automatically.

- Servo returns to its initial position.

Arduino Code:

#include <Servo.h>

Servo myServo;

int trigPin = 9;

int echoPin = 10;

int servoPin = 6;

long duration;

int distance;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

myServo.attach(servoPin);

myServo.write(0);   // Initial position

}

void loop() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

if (distance > 0 && distance < 10) {  // Hand detected

myServo.write(90);   // Press container

delay(1000);

myServo.write(0);    // Release

}

delay(500);

}

Conclusion: (Predefined/self-written)

The Automatic Hand Sanitizer Dispenser using Arduino, ultrasonic sensor, and servo motor demonstrates an effective contactless automation system.
It promotes hygiene and safety in public places and provides a practical example of sensor-based control systems.

For Your References:

[1] Arduino, Ultrasonic Sensor with Arduino, Arduino.cc

[2] Electronics Hub, “Automatic Hand Sanitizer Dispenser using Arduino,” Electronicshub.org, 2023. [Online]. Available: https://www.electronicshub.org/automatic-hand-sanitizer-dispenser/. [Accessed: 27-Aug-2025].