Automatic Room Light

Description:

A project using a Light Dependent Resistor (LDR) to detect light intensity and control an LED or light bulb accordingly.

Block Diagram:

Pinout / Circuit Connection:

LDR → Analog Input A0 on Arduino

10kΩ Resistor → connected with LDR for voltage divider

LED → Digital Pin 9 (with 220Ω resistor)

Components Required:

Arduino UNO

LDR

10kΩ Resistor

LED

220Ω Resistor

Breadboard

Jumper wires

Working Principle:

The LDR changes its resistance based on the light falling on it. Arduino reads the voltage from the voltage divider and switches the LED ON/OFF depending on threshold.

Operation (Step-by-Step):

Power the Arduino.

LDR senses ambient light.

Arduino compares LDR value with threshold.

If dark → LED turns ON.

If bright → LED turns OFF.

Arduino Code:

int ldrPin = A0;      // LDR connected to analog pin A0

int ledPin = 9;       // LED connected to digital pin 9

int threshold = 500;  // Light threshold (adjust if needed)

void setup() {

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int ldrValue = analogRead(ldrPin);   // Read light intensity

Serial.println(ldrValue);

if (ldrValue < threshold) {

// DARK condition (lights OFF)

digitalWrite(ledPin, HIGH);   // LED ON

} else {

// BRIGHT condition (lights ON)

digitalWrite(ledPin, LOW);    // LED OFF

}

delay(200);

}

Conclusion: (Predefined/self-written)

The LDR-based Light Sensor project teaches how sensors can automate lighting systems based on environmental conditions.

For Your References:

[1] Arduino.cc, 'Analog Read Reference,' https://www.arduino.cc/en/Reference/AnalogRead