LED Blink Using ESP32 with Wi-Fi

Description:

The LED Blink Using ESP32 with Wi-Fi project demonstrates how an ESP32 microcontroller can connect to a wireless network and control an LED.
In this project, the ESP32 connects to a Wi-Fi network and hosts a simple web server.
When accessed through a web browser, the server allows the user to turn the LED ON or OFF wirelessly.
This project introduces basic concepts of IoT, Wi-Fi communication, and remote device control.

Block Diagram:

Pinout / Circuit Connection:

- ESP32 GPIO 2 → LED Anode (via 220Ω resistor)

- LED Cathode → GND

- Power Supply → 5V (USB cable to ESP32)

Components Required:

- ESP32 Development Board

- 1 × LED

- 1 × 220Ω Resistor

- Jumper Wires

- Breadboard

- Wi-Fi Network

Working Principle:

The ESP32 has an inbuilt Wi-Fi module.
After connecting to a Wi-Fi network, the ESP32 runs a web server.
When a user accesses the ESP32 IP address through a browser, control commands are sent.
Based on these commands, the ESP32 changes the state of the GPIO pin, causing the LED to turn ON or OFF.

Operation (Step-by-Step):

• Connect the LED to the ESP32 GPIO pin using a resistor
• Upload the Wi-Fi LED control program to the ESP32
• Power ON the ESP32 using a USB cable
• Open the Serial Monitor to note the ESP32 IP address
• Enter the IP address in a web browser
• Click ON/OFF buttons to control the LED wirelessly

Arduino Code:

#include <WiFi.h>

#include <WebServer.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

WebServer server(80);

#define LED_PIN 2

void handleRoot() {

String html = "<h2>ESP32 LED Control</h2>";

html += "<a href=\"/on\"><button>LED ON</button></a><br><br>";

html += "<a href=\"/off\"><button>LED OFF</button></a>";

server.send(200, "text/html", html);

}

void handleOn() {

digitalWrite(LED_PIN, HIGH);

server.send(200, "text/html", "LED ON");

}

void handleOff() {

digitalWrite(LED_PIN, LOW);

server.send(200, "text/html", "LED OFF");

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nWiFi Connected");

Serial.println(WiFi.localIP());

server.on("/", handleRoot);

server.on("/on", handleOn);

server.on("/off", handleOff);

server.begin();

}

void loop() {

server.handleClient();

}

Conclusion: (Predefined/self-written)

The LED Blink Using ESP32 with Wi-Fi project demonstrates wireless control of electronic devices using IoT concepts.
It helps students understand ESP32 Wi-Fi capabilities, web server implementation, and GPIO control.
This project forms the foundation for advanced IoT applications such as smart home automation and remote monitoring.

For Your References:

[1] Espressif Systems, ESP32 Series Datasheet, Espressif Systems Inc., 2016.

[2] Arduino, “WiFi Library for ESP32,” Arduino.cc. [Online]. Available: https://www.arduino.cc/en/Reference/WiFi [Accessed: Jun. 12, 2018].

[3] N. Kolban, Kolban’s Book on ESP32, 1st ed. USA, 2017.

[4] Arduino, “Web Server with ESP32,” Arduino Project Hub, 2019.