# Ultrasonic Servo Radar Project

Description:

This project simulates a simple radar system using an ultrasonic sensor mounted on a servo motor. The servo rotates left and right while the ultrasonic sensor measures distance and scans the surroundings.

Block Diagram:

Pinout / Circuit Connection:

Servo Motor → PWM Pin (e.g., D9)
Ultrasonic Sensor Trig → D10
Ultrasonic Sensor Echo → D11
VCC and GND common for all components

Components Required:

- Arduino UNO
- Ultrasonic Sensor (HC-SR04)
- Servo Motor (SG90)
- Jumper Wires
- Breadboard (optional)

Working Principle:

The servo motor rotates from 0° to 180° while the ultrasonic sensor takes distance measurements. Each angle's distance is recorded to simulate radar scanning. The data can be viewed on the serial monitor or plotted.

Operation (Step-by-Step):

1. Connect the ultrasonic sensor to the Arduino.
2. Connect the servo motor to a PWM pin.
3. Upload the radar scanning code.
4. Servo rotates, sensor detects distance at each angle.
5. Output shown on Serial Monitor.

Arduino Code:

#include <Servo.h>
Servo s;
int trig = 10, echo = 11;
long duration; int distance;

void setup(){
  Serial.begin(9600);
  s.attach(9);
  pinMode(trig, OUTPUT);
  pinMode(echo, INPUT);
}

void loop(){
  for(int pos=0; pos<=180; pos+=10){
    s.write(pos);
    delay(200);
    distance = getDistance();
    Serial.print(pos);
    Serial.print("° : ");
    Serial.print(distance);
    Serial.println(" cm");
  }

  for(int pos=180; pos>=0; pos-=10){
    s.write(pos);
    delay(200);
    distance = getDistance();
    Serial.print(pos);
    Serial.print("° : ");
    Serial.print(distance);
    Serial.println(" cm");
  }
}

int getDistance(){
  digitalWrite(trig, LOW);
  delayMicroseconds(2);
  digitalWrite(trig, HIGH);
  delayMicroseconds(10);
  digitalWrite(trig, LOW);
  duration = pulseIn(echo, HIGH);
  return duration * 0.034 / 2;
}

Conclusion:

The Ultrasonic Servo Radar project demonstrates how servo rotation combined with ultrasonic sensing can be used to scan and map surroundings in a simple and effective way.