# Automatic Barrier Gate System

Description:

The Automatic Barrier Gate System is a simple automation project used to control a vehicle entry barrier. An ultrasonic sensor detects the presence of a vehicle near the gate. When a vehicle is detected within a certain distance, the Arduino sends a signal to a servo motor which lifts the barrier (straw) upward to allow the vehicle to pass. After a few seconds, the barrier automatically returns to its original position.

Block Diagram:

Pinout / Circuit Connection:

Ultrasonic Sensor (HC-SR04) → Arduino

| Ultrasonic Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | D9 |
| ECHO | D10 |

Servo Motor → Arduino

| Servo Wire | Arduino |
| --- | --- |
| Red | 5V |
| Brown/Black | GND |
| Orange/Yellow (Signal) | D6 |

Components Required:

- Arduino UNO

- Ultrasonic Sensor (HC-SR04)

- Servo Motor (SG90)

- Straw (used as barrier gate)

- Breadboard

- Jumper wires

- Power supply / USB cable

Working Principle:

The ultrasonic sensor measures the distance between the sensor and nearby objects using ultrasonic sound waves. When a vehicle approaches within a specific distance, the sensor detects it and sends the signal to the Arduino. The Arduino processes the signal and commands the servo motor to rotate and lift the barrier. After a short delay, the servo rotates back and lowers the barrier again.

Operation (Step-by-Step):

- Power ON the Arduino system.

- The ultrasonic sensor continuously measures the distance.

- When a vehicle approaches within the preset distance, the sensor detects it.

- Arduino sends a signal to the servo motor.

- The servo motor rotates and raises the straw barrier.

- After a few seconds, the barrier lowers automatically.

Arduino Code:

#include <Servo.h>

#define trigPin 9

#define echoPin 10

Servo barrierServo;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

barrierServo.attach(6);

Serial.begin(9600);

}

void loop() {

long duration;

int distance;

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.println(distance);

if (distance <= 15) {

barrierServo.write(90);   // Lift barrier

delay(3000);

} else {

barrierServo.write(0);    // Lower barrier

}

delay(500);

}

Conclusion:

The Automatic Barrier Gate System demonstrates a simple automation system used for vehicle entry control. By combining an ultrasonic sensor and servo motor, the system automatically opens and closes the barrier when a vehicle is detected. This concept is widely used in parking systems, toll gates, and restricted area entry systems.

References:

- Arduino Project Hub – Smart Dustbin using Ultrasonic Sensor.

- Electronics Tutorials – Servo Motor Interfacing with Arduino.