Mini Piano using Buzzer

Description:
This project creates a small electronic piano using an Arduino, push buttons, and a piezo buzzer. Each button represents a musical note such as C, D, E, F, or G. When pressed, the buzzer produces the corresponding frequency.

Block Diagram:

Pinout / Circuit Connection:

- Push buttons → Digital Pins 2–5

- Buzzer → Digital Pin 9

- Resistors for buttons (10kΩ pull-down)

Components Required:

- Arduino UNO

- Buzzer

- Push buttons (5)

- 10kΩ resistors

- Breadboard & jumper wires

Working Principle:
Each button is connected to a digital input pin. When pressed, the Arduino detects HIGH input and sends a PWM signal at a specific frequency to the buzzer. Each frequency corresponds to a musical note, e.g., 262 Hz for C, 294 Hz for D, etc. By pressing different buttons, different tones are generated, simulating a mini piano. The tone() function allows precise generation of square waves that drive the buzzer.
Operation (Step-by-Step):

- Connect buttons to Arduino inputs.

- Connect buzzer to D9.

- Upload code.

- Press different buttons → buzzer produces musical notes.

Code:

int buzzer = 9;

int switch1 = 2;

int switch2 = 3;

int switch3 = 4;

int switch4 = 5;

void setup() {

pinMode(buzzer, OUTPUT);

pinMode(switch1, INPUT_PULLUP);

pinMode(switch2, INPUT_PULLUP);

pinMode(switch3, INPUT_PULLUP);

pinMode(switch4, INPUT_PULLUP);

}

void loop() {

if (digitalRead(switch1) == LOW) {

tone(buzzer, 262);   // C note

}

else if (digitalRead(switch2) == LOW) {

tone(buzzer, 294);   // D note

}

else if (digitalRead(switch3) == LOW) {

tone(buzzer, 330);   // E note

}

else if (digitalRead(switch4) == LOW) {

tone(buzzer, 349);   // F note

}

else {

noTone(buzzer);      // Stop sound

}

}

Conclusion:
The mini piano project combines electronics and music in a simple yet educational way. It provides an engaging introduction to both hardware and coding, and can be extended to include more notes, polyphonic output, or even recording of tunes.

References:
[1] Arduino.cc, “Tone() – Arduino Reference,” 2023. [Online]. Available: https://www.arduino.cc/reference/en/language/functions/advanced-io/tone/. [Accessed: Sep. 13, 2025].
[2] M. Banzi and M. Shiloh, “Getting Started with Arduino,” 3rd ed., Maker Media, 2014.
. make file