Rain Alarm System

Description:

An electronic alarm system that detects rain using a rain sensor or metal plate electrodes and alerts via buzzer/LED.

Block Diagram:

Pinout / Circuit Connection:

- Rain Sensor Module/Metal Plate → Digital Input D2 on Arduino

- Buzzer → Digital Pin 8

Components Required:

- Arduino UNO

- Rain Sensor Module or DIY rain detector plate

- Buzzer

- 220Ω Resistor

- Jumper wires

- Breadboard

Working Principle:

When raindrops fall on the sensor plate, resistance changes, Arduino detects it and triggers buzzer/LED alarm.

Operation (Step-by-Step):

- Power ON the Arduino.

- Rain sensor detects rain drops.

- Arduino reads input from sensor.

- If rain detected → buzzer sounds.

Arduino Code:

#define RAIN_PIN 2

#define BUZZER_PIN 8

void setup() {

pinMode(RAIN_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(9600);

}

void loop() {

if (digitalRead(RAIN_PIN) == LOW) {

digitalWrite(BUZZER_PIN, HIGH);

Serial.println("Rain Detected");

} else {

digitalWrite(BUZZER_PIN, LOW);

Serial.println("No Rain");

}

delay(500);

}

Conclusion: (Predefined/self-written)

The Rain Alarm System provides a simple and effective way to detect rainfall and trigger alerts for applications like agriculture and household safety.

For Your References:

[1] Arduino Project Hub, 'Rain Sensor Module Tutorial,' https://projecthub.arduino.cc