Flame Detector

Description:

The Flame Detector project using Arduino is designed to detect the presence of fire or flame and provide an alert.
In this project, a flame sensor module is interfaced with an Arduino board to sense infrared radiation emitted by flames.
When a flame is detected, the system activates an alert device such as an LED or buzzer.
This project is useful for basic fire safety and monitoring applications.

Block Diagram:

Pinout / Circuit Connection:

Flame Sensor OUT → Arduino Digital Pin 7
Flame Sensor VCC → 5V
Flame Sensor GND → GND
LED / Buzzer → Arduino Digital Pin 13
Power supply: 5V (Arduino USB or adapter)

Components Required:

Arduino Uno / Nano
Flame Sensor Module
1 x Buzzer
Jumper wires
Breadboard

Working Principle:

The flame sensor detects infrared (IR) radiation emitted by fire.
When a flame is present, the sensor output changes and sends a digital signal to the Arduino.
The Arduino continuously monitors this signal.
If a flame is detected, the Arduino turns ON the LED or buzzer to provide an alert.
When no flame is present, the alert device remains OFF.

Operation (Step-by-Step):

- Connect the flame sensor to the Arduino as per the circuit diagram.

- Connect the Arduino to the computer using a USB cable.

- Open Arduino IDE and select the correct board and COM port.

- Upload the flame detector program to the Arduino.

- Power ON the Arduino.

- Bring a flame near the sensor.

- The sensor detects the flame and sends a signal to the Arduino.

- The LED or buzzer turns ON as an alert.

- When the flame is removed, the alert turns OFF.

Arduino Code:

int flamePin = 7;     // Flame sensor output

int alertPin = 13;   //  Buzzer

void setup() {

pinMode(flamePin, INPUT);

pinMode(alertPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int flameStatus = digitalRead(flamePin);

if (flameStatus == LOW) {   // Flame detected

digitalWrite(alertPin, HIGH);

Serial.println("Flame Detected!");

} else {

digitalWrite(alertPin, LOW);

Serial.println("No Flame");

}

delay(500);

}

Conclusion: (Predefined/self-written)

The Flame Detector using Arduino is a simple and effective safety project for detecting fire at an early stage.
It helps beginners understand sensor interfacing and digital input-output control.
This project can be extended by adding GSM, IoT modules, or automatic fire suppression systems.

For Your References:

[1]	Bharsakade et al., Advanced Fire & Gas Detection System, IJRaset.

[2]	Arduino, Digital Input and Output, Arduino.cc.