# Smart Dustbin

Description:

The Smart Dustbin is an automatic waste bin that opens its lid when a person brings their hand close to it. An ultrasonic sensor detects the presence of an object near the bin. The Arduino processes the sensor data and controls a servo motor to open the lid automatically. After a few seconds, the lid closes again. This project helps maintain hygiene and reduces the need to touch the dustbin manually.

Block Diagram:

Pinout / Circuit Connection:

Ultrasonic Sensor (HC-SR04) → Arduino

| Ultrasonic Pin | Arduino |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | D9 |
| ECHO | D10 |

Servo Motor → Arduino

| Servo Wire | Arduino |
| --- | --- |
| Red | 5V |
| Brown/Black | GND |
| Orange/Yellow (Signal) | D6 |

Components Required:

- Arduino UNO

- Ultrasonic Sensor (HC-SR04)

- Servo Motor (SG90)

- Cup or small container (used as dustbin)

- Jumper wires

- Breadboard

- Power supply / USB cable

Working Principle:

The ultrasonic sensor measures the distance between the sensor and any nearby object by sending ultrasonic waves and receiving the reflected signal. When a hand is placed near the bin, the sensor detects a short distance. Arduino then sends a signal to the servo motor to rotate and open the lid. After a few seconds, the servo rotates back and closes the lid automatically.

Operation (Step-by-Step):

- Power ON the Arduino system.

- The ultrasonic sensor continuously measures the distance.

- When a hand or object comes within a set distance (for example 15 cm), the sensor sends the data to Arduino.

- Arduino processes the signal and rotates the servo motor to open the dustbin lid.

- After a short delay, the servo motor rotates back to close the lid.

Arduino Code:

#include <Servo.h>

#define trigPin 9

#define echoPin 10

Servo myServo;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

myServo.attach(6);

Serial.begin(9600);

}

void loop() {

long duration;

int distance;

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.println(distance);

if (distance <= 15) {

myServo.write(90);   // open lid

delay(3000);

} else {

myServo.write(0);    // close lid

}

delay(500);

}

Conclusion:

The Smart Dustbin is an efficient and hygienic waste management system that automatically opens the lid when a person approaches. It demonstrates the use of ultrasonic sensors and servo motors for automation, making it a practical application of embedded electronics.

References:

- Arduino Project Hub – Smart Dustbin using Ultrasonic Sensor.

- Electronics Tutorials – Servo Motor Interfacing with Arduino.