ESP32 Digital Thermometer with OLED Display

Description:
This project builds a digital thermometer using an ESP32, a temperature sensor (e.g., DHT11/LM35), and an OLED display. The ESP32 reads the temperature and displays it in real-time. It demonstrates IoT concepts, sensor interfacing, and microcontroller display output, making it highly relevant for modern smart systems.
Block Diagram:

Pinout / Circuit Connection:

- DHT11/LM35 sensor OUT → GPIO 4

- OLED SDA → GPIO 21

- OLED SCL → GPIO 22

- VCC/GND → 3.3V and GND
Components Required:

- ESP32

- DHT11 or LM35 temperature sensor

- 0.96" OLED Display (I2C)

- Jumper wires
Working Principle:
The ESP32 reads analog/digital temperature values from the sensor. These values are processed and sent to the OLED display via I2C. The real-time temperature is updated continuously. With Wi-Fi capability, the ESP32 can also transmit the data online for IoT applications.
Operation (Step-by-Step):

- Connect sensor and OLED to ESP32.

- Upload thermometer code.

- View temperature on OLED display.

- (Optional) Extend by sending data to cloud.

Code:

// Using DHT11 sensor + SSD1306 OLED (I2C)

#include <Adafruit_SSD1306.h>

#include <Adafruit_GFX.h>

#include <DHT.h>

#include <Wire.h>

// OLED setup

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// DHT11 setup

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

// Initialize OLED

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED not found");

for (;;);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

// Initialize DHT sensor

dht.begin();

}

void loop() {

float temp = dht.readTemperature();   // Read Celsius

float hum  = dht.readHumidity();      // Read Humidity

// Check if sensor works

if (isnan(temp) || isnan(hum)) {

Serial.println("Sensor error!");

return;

}

// Print to Serial

Serial.print("Temperature: ");

Serial.print(temp);

Serial.print(" °C   Humidity: ");

Serial.print(hum);

Serial.println(" %");

// Print to OLED

display.clearDisplay();

display.setCursor(0, 10);

display.setTextSize(2);

display.print("Temp: ");

display.print(temp);

display.print(" C");

display.setCursor(0, 40);

display.setTextSize(2);

display.print("Hum: ");

display.print(hum);

display.print(" %");

display.display();

delay(2000); // Update every 2 seconds

}
Conclusion:
This project combines sensor interfacing, microcontroller programming, and display control, showcasing IoT-based monitoring systems. It’s a practical project with real-world applications such as weather stations and home automation.
References:
[1] R. Santos, “ESP32 OLED Display with Arduino IDE,” Random Nerd Tutorials, 2020. [Online]. Available: https://randomnerdtutorials.com/esp32-ssd1306-oled-display-arduino-ide/. [Accessed: Sep. 13, 2025].
[2] Espressif Systems, “ESP32 Technical Reference Manual,” 2019. [Online]. Available: https://www.espressif.com/sites/default/files/documentation/esp32_technical_reference_manual_en.pdf. [Accessed: Sep. 13, 2025].